SD5730 规格书	暖白光

◆Features:

◆Super high flux output and high luminance ◆Extremely wide viewing angle

◆Suitable for all SMT assembly and solder process ◆Available on tape and reel ◆Package:16K/reel

◆ROHS compliant

PART	NO.：SD5730NWN1

ATTENTION

OBSERVE PRECAUTONS

FOR HANDLING

ELECTROSTATIC

DISCHARGE

SENSITIVE

DEVICES

◆ABSOLUTE  MAXIMUM  RATING  （Ta=25℃）

| Item | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Peak Forward Current | IFp | 500 | mA |
| Reverse Voltage | VR | 5.0 | V |
| Continuous Forward Current | IF | 60 | mA |
| Recommend Operating Current | IF | 600 | mA |
| Operating Temperature | Topr | -40 to 85 | ℃ |
| Storage Temperature | Tstg | -40 to 105 | ℃ |
| Lead Soldering Temperature | Tsol | 250℃ For 3 Seconds |  |

◆CHARACTERISTICS (Ta=25℃)

|  | Parameter | Condition | Symbol |  |  | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Power Dissipation | IF=60mA | Pd |  |  | - | 0.2 | - | w |
|  | Correlated Color | IF=60mA | CCT |  |  | 2800 | - | 3000 | K |
|  | Temperature |  |  |  |  |  |  |  |  |
|  | Forward Voltage | IF=60mA | VF |  |  | 2.8 | 3.3 | 3.8 | v |
|  | Reverse Current | VR=5V | IR |  |  | / | / | 10 | μA |
|  | Chromaticity | IF=60mA | X |  |  | / | 0.31 | / |  |
|  | Coordinates |  |  |  |  |  |  |  |  |
|  |  | IF=60mA | Y |  |  | / | 0.33 | / |  |
|  | Luminous Flux | IF=60mA | φv |  |  | 30 | / | 40 | LM |
|  | Color Rending Index | IF=60mA | CRI |  |  | / | 70 | / | Ra |
|  | Full Viewing Angle | IF=60mA | 2θ1/2 |  |  | / | 120 | / | deg |
|  | ◆PACKAGE DIMENSIONS |  |  |  | ◆实物图 |  |  |  |  |

| REMARK：1:1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | 2013-10-15 | Page | 1/1 | Model No. | SD5630NWN1 | CHECKED |  |
| REV No. | A1 | Document No. | 1905637002 | CUSTOMER |  | APPROVED |  |

NOTE： Testing tolerance: VF±0.1V, IV(Φv) ±10%, 2θ1/2 ±10% ，CCT（λD）±15%;